Wildland Initial Size-Up Card

Cheat Sheet

Overview –

The intent of this size up card is to provide quick and relevant information to incoming resources helping to develop strategies, tactics, and resource requests.  A thorough fire size up with appropriate decision making can set the stage for the remainder of the incident.

This card has a front and back and can shrink for pocket size.  The front side is the necessary information for scene size up.  On a large escalating incident each category may be aired.  Open fields are intended for the user to take notes prior to airing size up.  Smaller, less complex incidents air only the categories that relate.  The back side provides recommendations and needs for consideration.

Location –

Provide confirmation of the location of the incident.  Often toned location or Reporting Party (R.P.) description does not provide accurate incident location.  Utilize common geographic features to help provide accurate information (Road intersection, property address, open space, drainage name)

- “You can show me arrival at the intersection of Arapahoe and N75th on a confirmed wildfire”

- “……… at 1133 Arapahoe Rd. on a confirmed wildfire”

- “……… at Shadow Canyon on a confirmed wildfire”

- “……… at Hall Ranch Open Space on a confirmed wildfire”

Approximate Size –

Provide a best guessed size.  Acreage vs. single tree.  Appx: 1 acre is the size of a football field (260’ x 160’).  A normal residential property is approximately 1/16 an acre.  This helps decide strategies, tactics, and potential resource requests.

| Small | ≥ ½ acre | Typically localized. Limited impact on environment or human health. Size could be characterized as .5 ac. 63.5% of the calls = small fires |
| --- | --- | --- |
| Medium | ½ acre – 10 acres | May cover specific areas but have limited impact on larger ecosystems. 24% of the calls = medium fires |
| Large | ≤ 10 acres | Significant area of coverage and environmental or human impacts. 2.7% of the calls = large fires |

- “We have a small…. Single tree”

- “Approximate 5 acres”

- “Approximate 1/10 acre”

Fuel Type –

What fuel type is the main carrier of the fire (what is burning).  This helps with fire behavior, strategies, tactics, and resource requests.   You may choose 1, 2 or all 3 but the idea is what is the main carrier.

| Grass | 1 hr. fuels ≥ ¼” dia. | Non woody herbaceous fine flashy fuels, groundcover, prairie, display highest rate of spread (Anderson FM 1,2, 3) |
| --- | --- | --- |
| Brush | 10 - 100 hr. fuels ¼” – R3” dia. | Fine woody matter, shrubbery, secondary overstory, light slash (Anderson FM 4,5,6,7) |
| Timber | 100 - 1000 hr. fuel 3” dia. above | Coniferous & Hardwoods, Standing or dead and down, slash, possible torching, spotting (Anderson FM 8,9,10) |

- “Burning in grass”

- “Burning in grass and brush”

- “Bruning in brush and timber”

Rate of Spread –

Describe how fast the fire is moving.  Creeping rate of spread will utilize the low category, if it’s running, moving at a high rate of speed you would use high.  This helps with fire behavior, fuel receptive, strategies, tactics, and resource request.

| Low | Creeping, short duration flame front, fire behavior can be predicted, little fire growth |
| --- | --- |
| Moderate | Short range spotting, mainly surface fire occasional single tree torching, some resistance to control measures. |
| High | Long range spotting, crown fire activity possible, difficult to control due to rapid fire growth. |

- “Low Rate of Spread”

- “Moderate rate of spread”

- “Hight rate of spread”

Potential –

What could be affected.  This describes the amount and size of fuel that could be affected.  This could be a large pocket of fuel (timber, brush) within the fire path, does not include structures.    Multiple pockets of larger material support extreme fire behavior warrant a high.  Patchy grass with no heavy fuel loading warrants a low.

| Low | Starts are unlikely, slow fire spread, low intensity, easy to extinguish, light to patchy minimal fuel loading. |
| --- | --- |
| Moderate | Higher potential for starts, flame length and fire behavior still allow for hand tools, heavier continuous fuel loading. Short range spotting |
| High | Starts are likely. Flame length and fire behavior exceed resource availability, large/heavy continuous fuel loading. Long range spotting |

- “Low potential” (patchy grass)

- “Moderate potential” (more continuous fuel loading)

- “High potential” (large/heavy continuous fuel loading)

Structures Threatened –

Are structures threatened?  Residential or outbuildings both apply.  Describe type of structure and amount threatened.  This helps with strategies, tactics, and resource requests.

| NO – | Nothing aired. No values at risk. Maybe values at risk in the vicinity but low rate of spread and potential don’t pose a threat. |
| --- | --- |
| YES – | Single or multiple values are at risk. Risk a little to save a lot. |

- “Multiple structures threatened”

- “No structures threatened”

- “We have no structures immediately threatened though 2 are in the area”

Actions –

What tactics are employed.  Actions should be planned and initiated based on current and expected fire behavior and situational demand.  Tactics include Direct, and Indirect (a distance away) though may provide more specific procedures.

| Direct Attack | Tactics on light fuels, low intensity fires (flame length ≤ 4’) enabling closer work to fires edge. (Suppression, progressive hose lay, anchor flank and pinch, Structure/Point Protection) |
| --- | --- |
| Indirect Attack | Tactic on hot fires with high rates of spread, containment lines are some distance from the fire edge. (Scouting, Evacuation, investigation, control lines, back firing, burn out, hand line) |

- “We will be going direct along the west flank”

- “We will be going indirect providing point protection to 1114 Flagstaff…….

- “We have a Knock on the fire and will be in mop-up……

Access / Stagging Area –

Access – provide best location for initial resource response and suppression assistance.

Staging Area – Provide adequate size and location for stagging multiple resource and/or personnel.  Base off projected incident duration, size, and resource response.  Request a radio frequency to manage resource demands from incident command.

- “Access will be off Forest Service Road 68J”

- “Access will be off Linden Dr.”

- “Staging area will be at Buckingham Park”

- “Staging area will be at the Kohls parking lot”

Communications –

Single/multiple channels needed.  Does the fire duration and number of resources overwhelm the current communication plan.  Boulder County Communications has provided two (2) VHF comm. plans.  Air the need for multiple channels early.  Pre-plan for potential incident within an incident.

- “We will need a command and tactical channel.”

- “I would like to request the use of B.C. comm plan 1.”

- “We will be working off company channel”

- Dispatch “……. you have been assigned FTAC12”

Resource Request –

Request adequate and specifically qualified resource support early.  Location, size, fuel type…. previous categories affect resource demand, strategies and tactics.  Request specific resource/apparatus per incident needs.

- “I would like to request 2 more brush trucks and a tender”.

- “I would like to request UTV w/ water and crew”

Command –

Name command of the incident.  Utilize common geographic features for naming ideas.  Does the incident lie within single or multi-jurisdictional boundaries (unified command).

Evacuations –

Notify early if fire behavior and effectiveness is inefficient to extinguish with resources on scene.  Utilize Boulder County Emergency Alert Request system to request specifics.

| No - | Nothing aired |
| --- | --- |
| Yes - | Utilize the Job Aid for requested action. Follow the process for severity level, area, and considerations. |

Water Sources –

Communicate water source locations, access, capacities, and types.  These should be adequate for effective suppression efforts.  Areas of concern without pressurized hydrants pose a threat, what other sources may be requested (tenders, water tanks, fold-a-tanks, pumpkins).

Law Enforcement –

Law Enforcement will assist with evacuations, road closures, animal control, and overall authority.  Delegate to minimize distraction and maintain focus on the incident priorities.

County FDO (Type 3 Team) –

If the initial agency has exceeded their capacity and capabilities of the escalating incident, request activation of the EOC and Boulder County Type 3 Incident Management Team.

Air Resources –

Consult with Boulder County F.D.O. on needs and specifics.  Coordination is a big part of effective incident management during escalating incidents.  Must have Latitude and Longitude, Radio Channel, Command Name, and Point of Contact prior to an order being filled.

Spot Weather Request –

When requested, an updated generalized forecast for the area is aired by dispatch from Denver Weather.  Should a incident require a spot weather request contact, Ft. Collins Interagency Dispatch Center direct with inputs for more detailed information.

Latitude & Longitude –

May assist with jurisdiction, command, potential financial details, etc.……